MDS5800 Software Release Notes	              Microwave Data Systems, Inc.

## Release 1.02

Release Date: 06/04/2004

### New Features Present in MDS5800 1.02 Firmware

- None

### Changes/Additions to Existing Features in MDS5800 1.02 Firmware

- None

### Defect Fixes in MDS5800 1.02 Firmware

- Important Product Note:
Version 1.02 contains bug fixes as described below.  It is recommended that all customers upgrade to 1.02.

- Problems Fixed

- The storage and retrieval of configuration information in the IDU has been made much more robust.  These changes will minimize the possibility of corruption of this information due to unexpected power failures or other events.

#### Known Errata and Limitations in MDS5800 1.02 Firmware

- Compatibility

- An IDU running 1.02 will function correctly if the IDU at the other end of the link is running version 0.79 or later software.

- The payload data encryption feature will only function if both IDU’s are running 1.01 or later, and both IDU serial numbers are CC0007600 or later.

- The version of iConductor used to connect to an IDU should always match the version of the software installed on that IDU.

## Release 1.01

Release Date: 04/19/2004

### New Features Present in MDS5800 1.01 Firmware

- Important Product Note

- Version 1.01 contains new features as described below.  It is recommended that all customers upgrade to 1.01

- General

- The MIB-2 variable ifInErrors for the payload over-air port now returns errored seconds rather than packet errors

- Added support for Ethernet payload Data Encryption

- Added new option to the SNMP variable antennaPolarization; It can now return external in addition to horizontal and vertical;

### Changes/Additions to Existing Features in MDS5800 1.01 Firmware

- None

### Defect Fixes in MDS5800 1.01 Firmware

- The SNMP MIB-2 variable ifSpeed now returns the actual speed of the over-the-air payload interface;  Previously it always returned 100 Mbits regardless of configuration mode of the IDU’s;

#### Known Errata and Limitations in MDS5800 1.01 Firmware

- Compatibility

- An IDU running 1.01 will function correctly if the IDU at the other end of the link is running version 0.79 or later software.

- The payload data encryption feature will only function if both IDU’s are running 1.01 and both IDU serial numbers are CC0007600 or later.

- SNMP

- MIB version 0.2.7 is required for this release

## Release 0.8

Release Date: 01/15/2004

### New Features Present in MDS5800 0.8 Firmware

- The following new features were added with this software release:

- Added the SNMP read-only variable erroredSeconds which shows the total number of errored seconds on the over-the-air link since the last time it was read.

- Added the SNMP read-only variable erroredSecondsTotal which shows the total number of errored seconds on the over-the-air link for the previous 24 hours.

- Added the SNMP read-only variable rohLinkState which shows whether the over-the-air radio overhead management link is up or down.

- Added the SNMP read-write variable overairNmsMode which is used to set or query the mode of the over-the-air network management link (Out-of-band, In-band, or Disabled).

- Added the SNMP read-only variable currentAlarmState which is used to determine whether the IDU has any active alarms or not.

- Added the SNMP read-only variable alarmCountTotal which shows the number of alarms that have occurred in the previous 24 hours.

- Added the Payload Disconnect alarm: If nothing is plugged into either payload port on an Ethernet IDU, this alarm will be set.

- Added the Shutdown IDU command to the Maintenance menu of the front panel interface.

### Changes/Additions to Existing Features in MDS5800 0.8 Firmware

- None.

### Defect Fixes in in MDS5800 0.8 Firmware

- Spanning Tree mode can no longer be enabled unless the Over-the-Air NMS mode is set to Out-of-Band.

#### Known Errata and Limitations in MDS5800 0.8 Firmware

- Compatibility

- An IDU running 0.8 will function correctly if the IDU at the other end of the link is running version 0.79 or later software.

- The payload data encryption feature will only function if both IDU’s are running 1.01 and both IDU serial numbers are CC0007600 or later.

- SNMP

- MIB version 0.2.4 is required for this release.