MDS NetView Installation Notes

Version 4.0

# New NetView Installation

## Installation from CD

1.1.1 After installing the CD in the CD drive, Windows should perform ‘autorun’ which will automatically execute ‘Launch.exe’ in the root directory.  If this does not happen, or ‘autorun’ is disabled, navigate to the CD root directory and double click ‘Launch.exe”.

## Installation from extracted zip file

### Download the NetView 4_0_1 zip file and extract the contents.  To install the application, double click ‘Launch.exe’.  Follow the instructions as the installation proceeds.

## NetView server:

### During the NetView Server installation, a dialog will prompt the user to enter the network name of the computer to which the Server is installed. The default is the name of the local machine.  If the network name is unknown or unavailable, it is recommended that the user leave the local computer name in the text box that is automatically entered by the installation.

### The decision to use Trap Tracker should be made prior to installation of NETview.  The NETview installation will automatically setup Trap Tracker and the NETview server to handle traps.

## Installation order: It does not matter the order of  installation of NETview server or client.  When the application is started, it does not matter which one is started first.  The installation sets them both up to find one another.

## TFTP server: After the TFTP installation, the correct path to the firmware files needs to be entered into the application in order for the TFTP server to operate correctly with NetView.  The TFTP server is installed in the Windows Desktop and is started by double clicking the TFTP icon.  Once started, the user can select the 'options' tab, and in each path field enter:  "C:\Program Files\Microwave Data Systems\MDSnv Server\tftp".

# Upgrade existing NetView Installation

## It is highly recommended that an earlier version of NETview be removed using the Windows removal program prior to installation of version 4.0.  This allows the easiest path for consistent results.

## Save existing database:  Since the data base may represent many hours of map development work, it is a good practice to create a backup prior to NetView upgrading.  The installation process will save any existing database, but in the event that the installation does not behave as expected, the investment of map creation should be saved.  The file is CMPdb.mdb and is located in C:\Program files\MicrowaveDataSystems\MDSnv Server\.  A copy of this file can be moved to another directory, or simply renamed.

IMPORTANT:

In order to give NETview 4.0 access to the maps saved from a previous version of NETview, the current database must be named CMPdb.mdb and reside in the directory:

C:\Program files\Microwave Data Systems\MDSnv Server\

The installation process must be allowed to add new tables to the existing database to be compatible with the 4.0 version.  Saving a copy of the database is a precaution against loosing map creation work.